ASSIGNMENT-8

Border Gateway Protocol (BGP)

Title: Configuration and Analysis of BGP Routing

- Objective

  - To understand the working of Border Gateway Protocol (BGP)

  - To configure External BGP (eBGP) between different Autonomous Systems

  - To analyze path selection using BGP attributes

- Theory

Border Gateway Protocol (BGP) is a path vector routing protocol used to exchange routing information between different Autonomous Systems (AS) on the Internet.

Key Concepts

  - Autonomous System (AS): A group of networks under a single administrative domain

  - eBGP: Routing between different AS

  - iBGP: Routing within the same AS

  - Path Attributes: Used to select best route

    - AS_PATH

    - NEXT_HOP

    - LOCAL_PREF

    - MED

BGP Characteristics

  - Uses TCP port 179

  - Supports policy-based routing

  - Ensures loop-free routing using AS_PATH

- Topology Description Network Setup:

  - 3 Routers (R1, R2, R3)

  - Each router belongs to a different AS

    - R1 → AS 100

    - R2 → AS 200

    - R3 → AS 300

  - Each router connected to a LAN

- IP Addressing Scheme

Device Interface IP Address  Subnet Mask

| R1 | G0/0 | 192.168.1.1 | 255.255.255.0 |
| --- | --- | --- | --- |
| R1 | G0/1 | 10.0.0.1 | 255.255.255.252 |
| R2 | G0/0 | 10.0.0.2 | 255.255.255.252 |
| R2 | G0/1 | 10.0.0.5 | 255.255.255.252 |
| R3 | G0/0 | 10.0.0.6 | 255.255.255.252 |
| R3 | G0/1 | 192.168.3.1 | 255.255.255.0 |

- Procedure

Step 1: Create Topology

  - Open Cisco Packet Tracer

  - Place 3 routers and connect using serial/ethernet links

  - Add PCs to each LAN

Step 2: Configure IP Addresses

Configure IP addresses on all router interfaces. Example (R1):

conf t interface g0/0

ip address 192.168.1.1 255.255.255.0

no shutdown interface g0/1

ip address 10.0.0.1 255.255.255.252

no shutdown

Step 3: Configure BGP

R1 Configuration (AS 100)

router bgp 100

neighbor 10.0.0.2 remote-as 200

network 192.168.1.0 mask 255.255.255.0

R2 Configuration (AS 200)

router bgp 200

neighbor 10.0.0.1 remote-as 100

neighbor 10.0.0.6 remote-as 300

network 10.0.0.0 mask 255.255.255.252

R3 Configuration (AS 300)

router bgp 300

neighbor 10.0.0.5 remote-as 200

network 192.168.3.0 mask 255.255.255.0

Step 4: Verify Configuration

Use the following commands: show ip bgp

show ip route

show ip bgp summary

Solution