# Assignment 6: Solution

# Routing using RIP Version 2 (Cisco Packet Tracer)

# Objective

To configure and verify routing using RIP Version 2 protocol in a network.

# Problem Statement

Design and implement the given network topology in Cisco Packet Tracer with three routers (R1, R2, R3), two LANs, and PCs using the provided IP addressing scheme.

| Network Details |  |  |  |
| --- | --- | --- | --- |
| Device | Interface | IP Address | Subnet Mask |
| R1 | S0/0/0 | 100.1.1.2 | 255.255.255.0 |
| R1 | S0/0/1 | 20.1.1.1 | 255.255.255.0 |
| R2 | S0/0/0 | 100.1.1.1 | 255.255.255.0 |
| R2 | G0/0 | 172.16.2.1 | 255.255.0.0 |
| R3 | S0/0/1 | 20.1.1.2 | 255.255.255.0 |
| R3 | G0/0 | 10.2.2.1 | 255.255.255.0 |
| PC1 | NIC | 172.16.2.10 | 255.255.0.0 |
| PC2 | NIC | 10.2.2.20 | 255.255.255.0 |
| Tasks |  |  |  |

- Create the network topology in Cisco Packet Tracer.

- Configure IP addresses on all router interfaces and PCs.

- Enable RIP Version 2 routing on all routers.

- Advertise all directly connected networks using RIP.

- Disable auto-summary in RIP configuration.

- Verify routing tables using 'show ip route'.

- Test connectivity by pinging PC2 from PC1 and between routers.

# Router Configurations

R1 Configuration:

enable conf t

interface s0/0/0

ip address 100.1.1.2 255.255.255.0

clock rate 64000 no shutdown interface s0/0/1

ip address 20.1.1.1 255.255.255.0 no shutdown

router rip version 2

no auto-summary

network 100.1.1.0

network 20.1.1.0 end

R2 Configuration:

enable conf t

interface s0/0/0

ip address 100.1.1.1 255.255.255.0

no shutdown interface g0/0

ip address 172.16.2.1 255.255.0.0 no shutdown

router rip version 2

no auto-summary network 100.1.1.0

network 172.16.0.0 end

R3 Configuration:

enable conf t

interface s0/0/1

ip address 20.1.1.2 255.255.255.0 no shutdown

interface g0/0

ip address 10.2.2.1 255.255.255.0 no shutdown

router rip version 2

no auto-summary network 20.1.1.0

network 10.2.2.0 end

Solution