Assignment 7

Routing using OSPF (Link State Routing) in Cisco Packet Tracer

Objective

- To understand the Link State Routing algorithm (OSPF)

- To configure and verify OSPF routing in a network

- To analyze how Shortest Path First (SPF) algorithm works

# Problem Statement

Design and implement the given network topology with three routers (R1, R2, R3), two LANs, and PCs.

In addition to configuration, analyze route selection using OSPF cost and SPF algorithm.

# Network Topology & IP Addressing

- 3 Routers: R1, R2, R3

- 2 LANs

- Serial links between routers

- PCs connected to R2 and R3

# Tasks

- Part A: Basic Configuration

  - Create the network topology in Cisco Packet Tracer.

  - Configure IP addresses on all router interfaces and PCs.

  - Verify connectivity between directly connected devices

# Part B: OSPF Configuration

- Enable OSPF on all routers: router ospf 1

- Configure router IDs manually (recommended): router-id 1.1.1.1

- Advertise networks using:

network <network-address> <wildcard-mask> area 0

- Ensure all routers are in Area 0 (Backbone Area)

# Sample Configuration R1

router ospf 1

router-id 1.1.1.1

network 100.1.1.0 0.0.0.255 area 0

network 20.1.1.0 0.0.0.255 area 0

# R2

router ospf 1

router-id 2.2.2.2

network 100.1.1.0 0.0.0.255 area 0

network 172.16.0.0 0.0.255.255 area 0

# R3

router ospf 1

router-id 3.3.3.3

network 20.1.1.0 0.0.0.255 area 0

network 10.2.2.0 0.0.0.255 area 0

# Part C: Link State (OSPF) Analysis (NEW) Step 1: Neighbor Discovery

- Use:

show ip ospf neighbor

- Identify OSPF neighbors for each router

# Step 2: Link State Database (LSDB)

- Use:

show ip ospf database

- Observe LSAs shared between routers Step 3: SPF Calculation (Conceptual)** Students must:

- Draw network graph

- Assign cost to each link (default or manual)

# Apply Shortest Path First (Dijkstra Algorithm)

- Part D: Verification Commands

- Check routing table: show ip route

- Check OSPF:

show ip ospf neighbor show ip ospf database

# Part E: Simulation & Testing

  - Ping from PC1 → PC2

  - Use: traceroute

  - Change bandwidth on one link: bandwidth 64

  - Observe route change based on cost

# Additional Analytical Task (NEW)

Fill the table:

| Router | Destination | Path | Cost |
| --- | --- | --- | --- |
| R1 | 10.2.2.0 | R1→R3 | ? |

Solution